CPE301 – SPRING 2019

# MIDTERM 2

Student Name: Eric Smith

Student #: 2000543940

Student Email: smithe4@unlv.nevada.edu

Primary Github address: https://github.com/smitheas95

Directory: C:\Users\eas7w\OneDrive\Documents\Repository\cpe301\Midterms\Midterm_2\Midterm2

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List:

ATMega328P XPLAINED MINI

APDS-9960

Wires (male-male, male-female)

WIFI – ESP8266MOD

Breadboard (provided from cpe310L lab kit)

- INITIAL/MODIFIED/DEVELOPED CODE OF MIDTERM 2

/*

midterm2.c

Created: 5/11/2019 3:37:39 PM

Author : Eric Smith

Q: Write, simulate, and demonstrate using Atmel Studio 7 a C code for the AVR

ATMEGA328p microcontroller that performs the following functions:

1. Program the I2C of ATmega328/p to read RGB/Ambient Light data from APDS

9960 sensor.

2. Display the value to UART.

3. Make sure the AT Firmware is downloaded into the ESP-01/ESP32 module.

4. Register for a free Thingspeak account with MATHWORK. Setup and get the

channel Key.

5. Transmit Lux sensor value to ESP-01/ESP32 through UART port using AT

Commands.

6. Display the Lux sensor value as a graph in Thingspeak

*/

#define F_CPU 16000000UL // DEFINE 16MHz

// NEEDED LIBRARIES

#include <avr/io.h>

#include <stdio.h>

#include <util/delay.h>

#include "uart.h"

#include "apds.h"

#include "i2c_master.h"

// BAUDRATE DEFINITIONS

#define BRGVAL (F_CPU/16/BAUD) - 1

#define BAUD 9600

// APDS DEFINITIONS

#define APDS_WRITE	(0x39 << 1) | 0

#define APDS_READ	(0x39 << 1) | 1

#ifndef APDS_H

#define APDS_H

#define APDS9960_I2C_ADDR       0x39 // I2C ADDRESS FOR APDS-9960

#define ERROR                   0xFF // RETURNED VALUES ERROR CODE

// MISC PARAMETERS

#define FIFO_PAUSE_TIME         30 // FIFO READS, WAIT PERIOD IN ms

// ACCEPTABLE DEVICE IDs

#define APDS9960_ID_1    0xAB

#define APDS9960_ID_2    0x9C

// BIT FIELDS

#define APDS9960_PON     0b00000001

#define APDS9960_AEN     0b00000010

#define APDS9960_PEN     0b00000100

#define APDS9960_WEN     0b00001000

#define APSD9960_AIEN    0b00010000

#define APDS9960_PIEN    0b00100000

#define APDS9960_GEN     0b01000000

#define APDS9960_GVALID  0b00000001

// APDS-9960 REGISTER ADDRESSES

#define APDS9960_ENABLE         0x80

#define APDS9960_ATIME          0x81

#define APDS9960_WTIME          0x83

#define APDS9960_PERS           0x8C

#define APDS9960_CONFIG1        0x8D

#define APDS9960_PPULSE         0x8E

#define APDS9960_CONFIG2        0x90

#define APDS9960_ID             0x92

#define APDS9960_RDATAL         0x96

#define APDS9960_RDATAH         0x97

#define APDS9960_GDATAL         0x98

#define APDS9960_GDATAH         0x99

#define APDS9960_BDATAL         0x9A

#define APDS9960_BDATAH         0x9B

#define APDS9960_POFFSET_UR     0x9D

#define APDS9960_POFFSET_DL     0x9E

#define APDS9960_CONFIG3        0x9F

// ON/OFF DEFINITIONS

#define OFF                     0

#define ON                      1

// ACCEPTABLE PARAMTERS FOR SETMODE

#define POWER                   0

#define AMBIENT_LIGHT           1

// DEFINE PROXIMITY

#define WAIT                    3

#define AMBIENT_LIGHT_INT       4

#define ALL                     7

// LED BOOST VALUES

#define LED_BOOST_100           0

#define LED_BOOST_150           1

#define LED_BOOST_200           2

#define LED_BOOST_300           3

// LED DRIVE VALUES

#define LED_DRIVE_100MA         0

#define LED_DRIVE_50MA          1

#define LED_DRIVE_25MA          2

#define LED_DRIVE_12_5MA        3

// DEFAULT VALUES

#define DEFAULT_ATIME           219 // 103 ms VALUE

#define DEFAULT_WTIME           246 // 27 ms VALUE

#define DEFAULT_PROX_PPULSE     0x87 // 16 us, PULSE OF 8

#define DEFAULT_POFFSET_UR      0 // OFFSET 0

#define DEFAULT_POFFSET_DL      0 // OFFSET 0

#define DEFAULT_CONFIG1         0x60 // WAIT FACTOR FOR NO 12x

#define DEFAULT_LDRIVE          LED_DRIVE_100MA

#define DEFAULT_PGAIN           PGAIN_4X

#define DEFAULT_AGAIN           AGAIN_4X

#define DEFAULT_AILT            0xFFFF // CALIBRATION FORCE INTERRUPT

#define DEFAULT_AIHT            0

#define DEFAULT_PERS            0x11 // 2 CONSECUTIVE PROX OR ALS FOR INT.

#define DEFAULT_CONFIG2         0x01 // SATURATION INTERRUPTS, LED BOOST NONE

#define DEFAULT_CONFIG3         0  // ENABLE ALL PHOTODIODES, NO SAI ENABLE ALL PHOTODIODES

#define DEFAULT_GLDRIVE         LED_DRIVE_100MA

#define DEFAULT_GWTIME          GWTIME_2_8MS

void apds_init(); // VOID FUNCTION FOR APDS INIT

void colorRead(); // VOID FUNCTION FOR READING COLOR

#endif

// FILE

FILE string_uart = FDEV_SETUP_STREAM(uart_putchar, NULL , _FDEV_SETUP_WRITE );

char results[256]; // 256

void init_UART(); // VOID FUNCTION FOR INIT UART

int uart_putchar( char c, FILE *stream); // VARIABLE INT FOR PUTTING CHARACTERS UART

int main(void) // INT MAIN & CONTAINS THINGSPEAK PORTION

{

uint16_t red = 0, green = 0, blue = 0; // SET RED, GREEN, BLUE TO ZERO

i2c_init();

init_UART();

stdout = &string_uart; // STRING UART

apds_init();

// RESPECTIVE DELAYS PROVIDED

_delay_ms(2500);

printf("AT\r\n");

_delay_ms(4500);

printf("AT+CWMODE=1\r\n");

_delay_ms(4500);

printf("AT+CWJAP=\"n/aVerizon-SM-G950U-748C\",\"n/anifj003)\"\r\n"); // WHERE I PUT MY HOTSPOT AND PASSWORD FOR HOTSPOT (ALTERED FOR SECURITY REASONS)

while (1) // KEY WHILE LOOP

{

_delay_ms(4500);

printf("AT+CIPMUX=0\r\n");

_delay_ms(4500);

printf("AT+CIPSTART=\"TCP\",\"api.thingspeak.com\",80\r\n");

_delay_ms(4500);

colorRead(&red, &green, &blue);

printf("AT+CIPSEND=104\r\n");

// API KEY WITH RESPECTIVE FIELDS FOR 3 COLORS

printf("GET https://api.thingspeak.com/update?api_key=PRH9WSZ8I5Y3A3LT&field1=%05u&field2=%05u&field3=%05u\r\n", red, green, blue);

_delay_ms(4500);

}

}

void init_UART(void){

// BAUD RATE SET

uint16_t baud_rate = BRGVAL;

UBRR0H = baud_rate >> 8;

UBRR0L = baud_rate & 0xFF;

// RECEIVER AND TRANSMITTER ENABLED

UCSR0B = ( 1 <<RXEN0)|( 1 <<TXEN0);

// FRAME FORMAT SET: 8 DATA, 1 STOP BIT

UCSR0C = (3 <<UCSZ00);

}

int uart_putchar(char c, FILE *stream){

// BUFFER IS EMPTY, WAIT TIL THEN

while ( !( UCSR0A & ( 1 <<UDRE0)) );

// BUFFER GETS DATA

UDR0 = c;

return 0;

}

void apds_init(){

uint8_t setup;

i2c_readReg(APDS_WRITE, APDS9960_ID, &setup,1);

if(setup != APDS9960_ID_1) while(1);

setup = 1 << 1 | 1<<0 | 1<<3 | 1<<4;

// I2C

i2c_writeReg(APDS_WRITE, APDS9960_ENABLE, &setup, 1);

setup = DEFAULT_ATIME;

i2c_writeReg(APDS_WRITE, APDS9960_ATIME, &setup, 1);

setup = DEFAULT_WTIME;

i2c_writeReg(APDS_WRITE, APDS9960_WTIME, &setup, 1);

setup = DEFAULT_PROX_PPULSE;

i2c_writeReg(APDS_WRITE, APDS9960_PPULSE, &setup, 1);

setup = DEFAULT_POFFSET_UR;

i2c_writeReg(APDS_WRITE, APDS9960_POFFSET_UR, &setup, 1);

setup = DEFAULT_POFFSET_DL;

i2c_writeReg(APDS_WRITE, APDS9960_POFFSET_DL, &setup, 1);

setup = DEFAULT_CONFIG1;

i2c_writeReg(APDS_WRITE, APDS9960_CONFIG1, &setup, 1);

setup = DEFAULT_PERS;

i2c_writeReg(APDS_WRITE, APDS9960_PERS, &setup, 1);

setup = DEFAULT_CONFIG2;

i2c_writeReg(APDS_WRITE, APDS9960_CONFIG2, &setup, 1);

setup = DEFAULT_CONFIG3;

i2c_writeReg(APDS_WRITE, APDS9960_CONFIG3, &setup, 1);

}

void colorRead(uint16_t *red, uint16_t *green, uint16_t *blue){

uint8_t redl, redh; // RED LOW, RED HIGH

uint8_t greenl, greenh; // GREEN LOW, GREEN HIGH

uint8_t bluel, blueh; // BLUE LOW, BLUE HIGH

i2c_readReg(APDS_WRITE, APDS9960_RDATAL, &redl, 1);

i2c_readReg(APDS_WRITE, APDS9960_RDATAH, &redh, 1);

i2c_readReg(APDS_WRITE, APDS9960_GDATAL, &greenl, 1);

i2c_readReg(APDS_WRITE, APDS9960_GDATAH, &greenh, 1);

i2c_readReg(APDS_WRITE, APDS9960_BDATAL, &bluel, 1);

i2c_readReg(APDS_WRITE, APDS9960_BDATAH, &blueh, 1);

*red = redh << 8 | redl;

*green = greenh << 8 | greenl;

*blue = blueh << 8 | bluel;

}

- DEVELOPED MODIFIED CODE OF MIDTERM 2

(NOT NEEDED, COVERED IN QUESTION #2)

- SCHEMATICS

Key schematic:

- SCREENSHOTS OF EACH TASK OUTPUT (THINGSPEAK OUTPUT)

Note: At time mark of 21:56, all colors spiked/went up when flashlight via phone was on the APDS sensor (can be observed in video link provided)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

Pictures provided below:

- VIDEO LINKS OF EACH DEMO

https://youtu.be/Ub53ePHh1fQ

- GITHUB LINK OF THIS DA

https://github.com/smitheas95/Midterm2

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Eric Smith

Key resources provided by professor (Dr. Venki):